UNCNCML110, UNCNCML111, UNCNCXL110, UNCNCEL110
CNC Central Unit
Reduced Data sheet

Version: 2

Date: 24.09.2026

# Overview

UNCNCML110, UNCNCXL110, UNCNCEL110 is a family of high performance CNC central unit capable of running complex CNC applications, and host HMI and third-parts CAD-CAM in Windows environment.

# Part Number

## Codes

| Code | Description |
| --- | --- |
| UNCNCML110 | Costantino CNC central unit, W11 IoT, intel i3 13th gen |
| UNCNCML111 | Costantino CNC central unit, W11 IoT, intel i3 13th gen, wifi option |
| UNCNCXL110 | Costantino CNC central unit, W11 IoT, intel i5 13th gen, wifi |
| UNCNCEL110 | Costantino CNC central unit, W11 IoT, intel i7 13th gen, wifi |

## Alternative codes

These products replace UNCNCML00x, UNCNCXL00x, UNCNCEL00x in new applications.

## Product status

| Item | Value |
| --- | --- |
| Status | In production |
| In production from: | Jan 2025 |
| In production at least until: | Jan 2030 |

# Technical data summary

## UNCNCML110/UNCNCML111

| Processor System |  |
| --- | --- |
| CPU | Intel® Core Processor 13th Gen Core™ i3-13100TE |
| Core Number | 4 |
| Max Speed | 4.1 GHz |
| Chipset | Intel® H610 |
| BIOS | AMI EFI 64 Mbit SPI BIOS |

## UNCNCXL110

| Processor System |  |
| --- | --- |
| CPU | Intel® Core Processor 13th Gen Core™ i5-13100TE |
| Core Number | 12 (8 P, 4 E) |
| Max Speed | 4.1 GHz |
| Chipset | Intel® H610 |
| BIOS | AMI EFI 64 Mbit SPI BIOS |

## UNCNCEL110

| Processor System |  |
| --- | --- |
| CPU | Intel® Core Processor 13th Gen Core™ i7-13100TE |
| Core Number | 16 (8 P, 4 E) |
| Max Speed | 4.1 GHz |
| Chipset | Intel® H610 |
| BIOS | AMI EFI 64 Mbit SPI BIOS |

## Common features

| Operating System |  |
| --- | --- |
|  | Microsoft Windows® 11 IoT Enterprise 2024 H2 LTSC |
| CNC |  |
| Costantino | SWCNCCOST6 |
| Default Axes Number | 3 interpolated |
| Max Axes Number | 12 interpolated + 4 independent (to be enabled w/ software options) |
| Max Channels Number | 4 (to be enabled w/ software option) |
| Min Interpolation Time | 0.5 ms |
| Throughput | 64,000 blocks/sec (1 channel w/ dedicated core, 3 axes CNC) |
| Part Program | ISO 6983 (G-code) |
| PLC |  |
| Standard | IEC 61131 (KW Multiprog) Ansi C |
| Memory |  |
| Technology | DDR5 4800 Dual Channel |
| Size | 8 GB |
| Video |  |
| Graphics Controller | Intel® UHD Graphics 730 |
| Max Resolution (HDMI) | 4096 x 2160 @ 60Hz |
| Max Resolution (DP) | 7680 x 4320 @ 60Hz |
| Display Memory | Shared |
| Physical Connections | 1x Display Port V1.4 1x HDMI V2.0 |
| Storage |  |
| Size | 128GB |
| Flash | SSD 112L 3D |
| Interface | PCIe/NVME |
| Form Factor | M.2 2242 |
| Max Data Transfer Rate | 3100/1700MB/s (R/W) |
| Network |  |
| Ethernet 1 controller | Intel i219LM with 10/100/1000 Mbit/s |
| Ethernet 2 controller | Realtek RTL8111H with 10/100/1000 Mbit/s |
| Ethernet 3 controller (**) | Microchip LAN9500 with 10/100 Mbit/s |
| Physical Connections | 3x RJ45 shielded |
| Wi-Fi (**) |  |
| Supported Standard | IEEE 802.11 b/g/n |
| Frequency Band | 2.4 GHz |
| Security | WEP 64/128, WPA,WPA2, TKIP,AES |
| Field Bus |  |
| EtherCAT controller | Intel i210 |
| Physical Connections | 1x RJ45 shielded |
| Peripherals |  |
| USB | USB 2.0 USB 3.2 |
| Physical Connections | 4x USB 2.0 type-A 1x USB 3.2 type-A 1x USB 3.2 type-C |
| Serial Port | COM RS-485 (Modbus RTU) |
| Physical Connections | 1x D-Sub 9 Male |
| Power Requirements |  |
| Nominal Input Voltage | Single Voltage 24V DC |
| Max Power Consumption | 80W |
| Power On | AT (Directly Power on as power input ready) |
| Max Inrush Current | 4A (5ms) |
| Physical Connections | 1x Removable Terminal Block |
| Cooling |  |
| Fan | #2 60mm diameter replaceable |
| MTBF | 70Kh |
| Environment |  |
| Operating Temperature | 0 ~ 45 °C |
| Storage Temperature | -20 ~ 70 °C |
| Humidity | Up to 90% without condensation |
| Overall Dimension | 192 x 57 x 184 mm (HxWxD) |
| Weight | 2 Kg |

(**) The WI-FI feature is not available for UNCNCML110

# Certifications

| Certification | Description |
| --- | --- |
| CE | EU Declaration of Conformity: EMC Directive 2014/30/UE Standards used: EN 61000-6-2:2019 EN 61000-6-4:2019 |
| RoHS | Declaration of Conformity: RoHS Directive 2011/65/EU |
| Origin | Italy |
| CRA | Yes (pending) |

# Layout and dimension

| Height | 192,0 mm |
| --- | --- |
| Width | 57,0 mm |
| Depth | 184,0 mm |

Drilling Template for plate 1

Drilling Template for plate 2 (Optional)

Mechanical drawing

# Reference documents

Full technical data sheet: TDU2001_UNCNCML110, TDU2002_UNCNCXL110, TDU2003_UNCNCEL110.

These products have a comparison performance evaluation in TABRD001_validationLists document.